# Planning Wizard - Calculation Formulas

This document contains all the formulas used in the Planning Wizard for technical sizing, economic analysis, and emissions calculations.

──────────────────────────────────────────────────

## 1. TECHNICAL SIZING FORMULAS

### 1.1 Basic Constants and Load Calculations

Local Load (kW)

local_load = total_daily_energy_kwh / 24

Day/Night Energy Split

day_time_energy = total_daily_energy_kwh × (7.03 / 13.01)
night_time_energy = total_daily_energy_kwh × (1 - 7.03 / 13.01)

Solar Insolation

daily_solar_insolation = 5.02 kWh/m²/day

──────────────────────────────────────────────────

### 1.2 Efficiency Factors

Component Efficiencies:

`n_module = 0.97` - Module efficiency

`n_temperature = 0.89` - Temperature derating

`n_dust = 0.95` - Dust derating

`n_inverter = 0.9` - Inverter efficiency

`n_dcdc = 0.95` - DC-DC converter efficiency

`n_charging = 0.9` - Battery charging efficiency

`n_discharging = 0.9` - Battery discharging efficiency

Combined Efficiencies:

n_panel = n_module × n_temperature × n_dust = 0.97 × 0.89 × 0.95 = 0.8207
n_panel = 1  # Flask explicitly overrides to 1 (used in all calculations)
n_battery_roundtrip = n_charging × n_discharging = 0.9 × 0.9 = 0.81

⚠️ IMPORTANT: In all formulas below, n_panel = 1 is used (not the calculated 0.8207). This matches the Flask implementation exactly.

Battery Depth of Discharge:

battery_dod = 0.8 (80%)

Battery Nominal Voltage:

battery_nominal_voltage = 12 V

──────────────────────────────────────────────────

### 1.3 Solar Panel Sizing

Energy Generated by PV System:

# Note: n_panel = 1 (Flask override)
pv_energy_gen = (day_time_energy / (n_panel × n_inverter × n_dcdc)) +
                (night_time_energy / (n_panel × n_inverter × n_dcdc × n_battery_roundtrip))
pv_energy_gen = (day_time_energy / (1 × 0.9 × 0.95)) +
                (night_time_energy / (1 × 0.9 × 0.95 × 0.81))
pv_energy_gen = (day_time_energy / 0.855) + (night_time_energy / 0.69255)

Peak Power of PV System:

pv_peak_power = pv_energy_gen / daily_solar_insolation

Example:

If `day_time_energy = 27.03 kWh` and `night_time_energy = 22.97 kWh`

`pv_energy_gen = (27.03 / (1 × 0.9 × 0.95)) + (22.97 / (1 × 0.9 × 0.95 × 0.81))`

`pv_energy_gen = (27.03 / 0.855) + (22.97 / 0.69255) = 31.61 + 33.17 = 64.78 kWh`

`pv_peak_power = 64.78 / 5.02 = 12.90 kW`

──────────────────────────────────────────────────

### 1.4 Battery Sizing

Battery Energy (kWh):

# Note: n_panel = 1 (Flask override)
battery_energy = night_time_energy / (n_panel × n_inverter × n_dcdc × n_battery_roundtrip × battery_dod)
battery_energy = night_time_energy / (1 × 0.9 × 0.95 × 0.81 × 0.8)
battery_energy = night_time_energy / 0.55404

Battery Capacity (kAh):

battery_capacity = battery_energy / battery_nominal_voltage
battery_capacity = battery_energy / 12

Example:

If `night_time_energy = 22.97 kWh`

`battery_energy = 22.97 / (1 × 0.9 × 0.95 × 0.81 × 0.8) = 22.97 / 0.55404 = 41.45 kWh`

`battery_capacity = 41.45 / 12 = 3.45 kAh`

──────────────────────────────────────────────────

### 1.5 Inverter & DC-DC Converter Sizing

Inverter Rating (kVA):

inverter_rating = pv_peak_power × 1.25

DC-DC Converter Rating (kW):

dcdc_sizing = inverter_rating

──────────────────────────────────────────────────

### 1.6 Grid Connection Sizing

Grid Connection (kW):

grid_connection_kw = peak_load_kw = total_daily_energy_kwh / 8

(Assumes 8 hours of peak usage)

──────────────────────────────────────────────────

### 1.7 Diesel Generator Sizing (Optional)

Diesel Capacity (kW):

diesel_capacity_kw = local_load × 1.1

(Only if allow_diesel = True and "diesel" in preferred_sources)

──────────────────────────────────────────────────

## 2. ECONOMIC ANALYSIS FORMULAS

### 2.1 Cost Constants

Component Unit Costs (derived from Flask code):

pv_cost_perkW = 17,846,400 / 594.92 = ₹30,004.81 per kW
battery_cost_per_kWh = 17,720,000 / (177.2 × 12) = ₹8,333.33 per kWh
inverter_cost_per_kVA = 5,949,200 / 743.65 = ₹8,001.88 per kVA
dcdc_cost_per_kW = 991,650 / 743.65 = ₹1,333.65 per kW

Additional Component Costs (per kVA/kWh):

add_cap_inv_dual_mode_per_kVA = 1,984,000 / 743.65 = ₹2,668.62 per kVA
add_sc_inv_dual_mode_per_kVA = 2,976,000 / 743.65 = ₹4,002.93 per kVA
add_cap_inv_on_grid_per_kVA = 992,000 / 743.65 = ₹1,334.31 per kVA
add_sc_inv_on_grid_per_kVA = 1,388,000 / 743.65 = ₹1,866.90 per kVA

add_cap_bat_dual_mode_per_kWh = 119,000 / (12 × 177.2) = ₹55.97 per kWh
add_sc_bat_dual_mode_per_kWh = 138,800 / (12 × 177.2) = ₹65.30 per kWh

Installation & O&M:

Installation cost: 10% of equipment cost

Annual O&M cost: 3% of (PV + Battery) cost

Interest Rate:

annual_interest_rate = 0.1 (10%)

──────────────────────────────────────────────────

### 2.2 Component Cost Calculations

Solar Panel Cost:

pv_cost = pv_cost_perkW × pv_peak_power

Battery Cost:

battery_cost = battery_cost_per_kWh × battery_energy

Inverter Cost:

inverter_cost = inverter_cost_per_kVA × inverter_rating

DC-DC Converter Cost:

dcdc_cost = dcdc_cost_per_kW × dcdc_sizing

──────────────────────────────────────────────────

### 2.3 Dual Mode System Costs

Installation Cost:

Installation_cost_dual_mode = 0.1 × (pv_cost + battery_cost)

Annual O&M Cost:

annual_om_cost_dual_mode = 0.03 × (pv_cost + battery_cost)

Capital Cost:

capital_cost_dual_mode = pv_cost + battery_cost + inverter_cost + dcdc_cost + Installation_cost_dual_mode

Annual Cost (including interest):

annual_cost_dual_mode = capital_cost_dual_mode × annual_interest_rate + annual_om_cost_dual_mode

──────────────────────────────────────────────────

### 2.4 On-Grid System Costs

Installation Cost:

Installation_cost_on_grid = 0.1 × pv_cost

Annual O&M Cost:

annual_om_cost_on_grid = 0.03 × pv_cost

Capital Cost:

capital_cost_on_grid = pv_cost + inverter_cost + Installation_cost_on_grid

Annual Cost (including interest):

annual_cost_on_grid = capital_cost_on_grid × annual_interest_rate + annual_om_cost_on_grid

──────────────────────────────────────────────────

### 2.5 Annual Energy Generation

Solar Insolation Constants for Outage Scenarios:

daily_solar_insolation = 5.02 kWh/m²/day
one_hour_outage_solar_insolation = 4.6 kWh/m²/day
two_hour_outage_solar_insolation = 4.2 kWh/m²/day
three_hour_outage_solar_insolation = 3.8 kWh/m²/day

On-Grid System - Normal Operation (No Outage):

# Note: The "1" in the formula represents n_panel = 1 (Flask override)
T_on = 365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter
T_on = 365 × pv_peak_power × 5.02 × 1 × 0.9
T_on = 365 × pv_peak_power × 5.02 × 0.9

Dual Mode System - Normal Operation (No Outage):

# Note: The "1" in the formula represents n_panel = 1 (Flask override)
T_dual = 365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter × n_dcdc
T_dual = 365 × pv_peak_power × 5.02 × 1 × 0.9 × 0.95
T_dual = 365 × pv_peak_power × 5.02 × 0.9 × 0.95

──────────────────────────────────────────────────

### 2.6 Cost of Energy Generation (Normal Operation)

On-Grid System:

cost_energy_on_grid = annual_cost_on_grid / T_on

Dual Mode System:

cost_energy_dual_mode = annual_cost_dual_mode / T_dual

──────────────────────────────────────────────────

### 2.7 Annual Revenue

Revenue Rate:

revenue_per_kWh = ₹8.0

Annual Revenue:

annual_revenue_on_grid = T_on × 8
annual_revenue_dual_mode = T_dual × 8

──────────────────────────────────────────────────

### 2.8 Cost of Energy Generation (Grid Outage Scenarios)

#### 2.8.1 On-Grid System - Daytime Outage

Energy Generation During Outage:

T_on_onehour = 365 × pv_peak_power × one_hour_outage_solar_insolation × 1 × n_inverter
T_on_twohour = 365 × pv_peak_power × two_hour_outage_solar_insolation × 1 × n_inverter
T_on_threehour = 365 × pv_peak_power × three_hour_outage_solar_insolation × 1 × n_inverter

Cost of Energy:

cost_energy_on_grid_onehour = annual_cost_on_grid / T_on_onehour
cost_energy_on_grid_twohour = annual_cost_on_grid / T_on_twohour
cost_energy_on_grid_threehour = annual_cost_on_grid / T_on_threehour

#### 2.8.2 On-Grid System - Nighttime Outage

Energy Generation (unchanged):

T_on_night = 365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter

Cost of Energy:

cost_energy_on_grid_night = annual_cost_on_grid / T_on_night

#### 2.8.3 Dual Mode System - Daytime Outage (No Outage = 0 hours)

Energy Generation:

T_dual_zero = 365 × ((daily_solar_insolation × local_load) +
                     (pv_peak_power - (local_load / (n_dcdc × n_inverter))) ×
                     daily_solar_insolation × n_dcdc × n_inverter)

#### 2.8.4 Dual Mode System - Daytime Outage (1, 2, 3 hours)

Energy Generation:

T_dual_onehour = 365 × ((daily_solar_insolation × local_load) +
                        (pv_peak_power - (local_load / (n_dcdc × n_inverter))) ×
                        one_hour_outage_solar_insolation × n_inverter × n_dcdc)

T_dual_twohour = 365 × ((daily_solar_insolation × local_load × 1) +
                        (pv_peak_power × 1 - (local_load / (n_dcdc × n_inverter × 1))) ×
                        two_hour_outage_solar_insolation × n_inverter × n_dcdc)

T_dual_threehour = 365 × ((daily_solar_insolation × local_load × 1) +
                          (pv_peak_power × 1 - (local_load / (n_dcdc × n_inverter × 1))) ×
                          three_hour_outage_solar_insolation × n_inverter × n_dcdc)

Annual Secondary Benefits (Savings from avoiding diesel):

AS_one_hour = local_load × 365 × 1 × 29  (₹29 per kWh saved)
AS_two_hour = local_load × 365 × 2 × 29
AS_three_hour = local_load × 365 × 3 × 29

Cost of Energy:

cost_energy_dual_mode_onehour = (annual_cost_dual_mode - AS_one_hour) / T_dual_onehour
cost_energy_dual_mode_twohour = (annual_cost_dual_mode - AS_two_hour) / T_dual_twohour
cost_energy_dual_mode_threehour = (annual_cost_dual_mode - AS_three_hour) / T_dual_threehour

#### 2.8.5 Dual Mode System - Nighttime Outage

Energy Generation:

T_dual_night_onehour = 365 × (local_load × (daily_solar_insolation × 1 + 1) +
                              daily_solar_insolation × n_dcdc × n_inverter ×
                              (pv_peak_power × 1 - (local_load / (n_dcdc × n_inverter × 1))))

T_dual_night_twohour = 365 × (local_load × 1 × (daily_solar_insolation + 2) +
                              daily_solar_insolation × n_dcdc × n_inverter ×
                              (pv_peak_power × 1 - (local_load / (n_dcdc × n_inverter × 1))))

T_dual_night_threehour = 365 × (local_load × 1 × (daily_solar_insolation + 3) +
                                daily_solar_insolation × n_dcdc × n_inverter ×
                                (pv_peak_power × 1 - (local_load / (n_dcdc × n_inverter × 1))))

Annual Battery Cost (Grid charging for nighttime outage):

Annual_battery_cost_onehour = local_load × 365 × 1 × 8  (₹8 per kWh from grid)
Annual_battery_cost_twohour = local_load × 365 × 2 × 8
Annual_battery_cost_threehour = local_load × 365 × 3 × 8

Cost of Energy:

cost_energy_dual_mode_night_onehour = (annual_cost_dual_mode + Annual_battery_cost_onehour - AS_one_hour) / T_dual_night_onehour
cost_energy_dual_mode_night_twohour = (annual_cost_dual_mode + Annual_battery_cost_twohour - AS_two_hour) / T_dual_night_twohour
cost_energy_dual_mode_night_threehour = (annual_cost_dual_mode + Annual_battery_cost_threehour - AS_three_hour) / T_dual_night_threehour

──────────────────────────────────────────────────

### 2.9 Simple Payback Period

Dual Mode System:

Simple_payback_dual_mode = capital_cost_dual_mode / (annual_revenue_dual_mode - annual_om_cost_dual_mode)

On-Grid System:

Simple_payback_on_grid = capital_cost_on_grid / (annual_revenue_on_grid - annual_om_cost_on_grid)

──────────────────────────────────────────────────

## 3. CARBON EMISSIONS CALCULATION FORMULAS

### 3.1 Constants

System Constants:

daily_solar_insolation = 5.02 kWh/m²/day
n_inverter = 0.9
n_dcdc = 0.95

Emission Factor:

emission_factor = 0.8 × 30 / 1000 = 0.024 tonnes CO₂ per kWh

──────────────────────────────────────────────────

### 3.2 Annual Energy Generation

Dual Mode System:

# Note: The "1" in the formula represents n_panel = 1 (Flask override)
T_dual = 365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter × n_dcdc
T_dual = 365 × pv_peak_power × 5.02 × 1 × 0.9 × 0.95
T_dual = 365 × pv_peak_power × 5.02 × 0.9 × 0.95

On-Grid System:

# Note: The "1" in the formula represents n_panel = 1 (Flask override)
T_on = 365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter
T_on = 365 × pv_peak_power × 5.02 × 1 × 0.9
T_on = 365 × pv_peak_power × 5.02 × 0.9

──────────────────────────────────────────────────

### 3.3 Carbon Emission Calculation

Dual Mode System:

Carbon_emmission_dual_mode (tonnes) = T_dual × 0.8 × 30 / 1000
Carbon_emmission_dual_mode (tonnes) = T_dual × 0.024

On-Grid System:

Carbon_emmission_on_grid (tonnes) = T_on × 0.8 × 30 / 1000
Carbon_emmission_on_grid (tonnes) = T_on × 0.024

──────────────────────────────────────────────────

## 4. EXAMPLE CALCULATIONS

### Example 1: Technical Sizing

Input:

`total_daily_energy_kwh = 50.0 kWh/day`

Step 1: Calculate Local Load

local_load = 50.0 / 24 = 2.083 kW

Step 2: Calculate Day/Night Split

day_time_energy = 50.0 × (7.03 / 13.01) = 27.03 kWh
night_time_energy = 50.0 × (1 - 7.03 / 13.01) = 22.97 kWh

Step 3: Calculate PV Energy Generation

# Note: n_panel = 1 (Flask override), not 0.8207
n_panel = 1
n_battery_roundtrip = 0.9 × 0.9 = 0.81

pv_energy_gen = (27.03 / (1 × 0.9 × 0.95)) + (22.97 / (1 × 0.9 × 0.95 × 0.81))
pv_energy_gen = (27.03 / 0.855) + (22.97 / 0.69255)
pv_energy_gen = 31.61 + 33.17 = 64.78 kWh

Step 4: Calculate PV Peak Power

pv_peak_power = 64.78 / 5.02 = 12.90 kW

Step 5: Calculate Battery Energy

battery_energy = 22.97 / (1 × 0.9 × 0.95 × 0.81 × 0.8)
battery_energy = 22.97 / 0.55404 = 41.45 kWh

Step 6: Calculate Battery Capacity (kAh)

battery_capacity = 41.45 / 12 = 3.45 kAh

Step 7: Calculate Inverter Rating

inverter_rating = 12.90 × 1.25 = 16.13 kVA

Step 8: Calculate DC-DC Converter Rating

dcdc_sizing = 16.13 kW

──────────────────────────────────────────────────

### Example 2: Economic Analysis (Dual Mode System)

Input (from Example 1):

`pv_peak_power = 12.90 kW`

`battery_energy = 41.45 kWh`

`inverter_rating = 16.13 kVA`

`dcdc_sizing = 16.13 kW`

Step 1: Component Costs

pv_cost = 12.90 × 30,004.81 = ₹387,062.05
battery_cost = 41.45 × 8,333.33 = ₹345,416.67
inverter_cost = 16.13 × 8,001.88 = ₹129,030.32
dcdc_cost = 16.13 × 1,333.65 = ₹21,511.77

Step 2: Installation Cost

Installation_cost_dual_mode = 0.1 × (387,062.05 + 345,416.67) = ₹73,247.87

Step 3: Capital Cost

capital_cost_dual_mode = 387,062.05 + 345,416.67 + 129,030.32 + 21,511.77 + 73,247.87
capital_cost_dual_mode = ₹956,268.68

Step 4: Annual O&M Cost

annual_om_cost_dual_mode = 0.03 × (387,062.05 + 345,416.67) = ₹21,974.36

Step 5: Annual Cost (with interest)

annual_cost_dual_mode = 956,268.68 × 0.1 + 21,974.36 = ₹117,601.23

Step 6: Annual Energy Generation

T_dual = 365 × 12.90 × 5.02 × 1 × 0.9 × 0.95 = 20,183.29 kWh

Step 7: Cost of Energy Generation

cost_energy_dual_mode = 117,601.23 / 20,183.29 = ₹5.83 per kWh

Step 8: Annual Revenue

annual_revenue_dual_mode = 20,183.29 × 8 = ₹161,466.32

Step 9: Simple Payback Period

Simple_payback_dual_mode = 956,268.68 / (161,466.32 - 21,974.36)
Simple_payback_dual_mode = 956,268.68 / 139,491.96 = 6.85 years

──────────────────────────────────────────────────

### Example 3: Carbon Emissions

Input (from Example 1):

`pv_peak_power = 12.90 kW`

`T_dual = 20,183.29 kWh` (from Example 2)

`T_on = 365 × 12.90 × 5.02 × 1 × 0.9 = 21,245.53 kWh`

Dual Mode Carbon Emissions:

Carbon_emmission_dual_mode = 20,183.29 × 0.024 = 484.40 tonnes

On-Grid Carbon Emissions:

Carbon_emmission_on_grid = 21,245.53 × 0.024 = 509.89 tonnes

──────────────────────────────────────────────────

## 5. NOTES AND DISCLAIMERS

### 5.1 Formula Notes

**Panel Efficiency (n_panel):** The Flask code explicitly sets `n_panel = 1` (overriding the calculated value of 0.8207) and uses `n_panel = 1` in **ALL** calculations. This means:

`n_panel = n_module × n_temperature × n_dust` is calculated first (= 0.8207)

Then `n_panel = 1` is explicitly assigned (Flask override)

All subsequent formulas use `n_panel = 1`, not 0.8207

The "1" appears explicitly in formulas like `365 × pv_peak_power × daily_solar_insolation × 1 × n_inverter`

**Outage Scenarios:** The formulas for dual mode system during outages include complex calculations that account for:

Battery backup during outages

Solar generation reduction during daytime outages

Grid charging requirements for nighttime outages

Secondary benefits from avoiding diesel generator use

**Cost Calculations:** Component costs are derived from the Flask code's total project costs divided by installed capacity, providing realistic unit costs.

**Simple Payback Period:** Uses net revenue (revenue minus O&M costs) rather than gross revenue, providing a more conservative estimate.

### 5.2 Implementation Details

All calculations match the Flask `solar-battery-calculation` endpoint exactly

Results are rounded to 2 decimal places for display

The implementation includes both dual mode and on-grid system calculations for comparison

All outage scenarios (daytime and nighttime, 0-3 hours) are calculated and included in the response

──────────────────────────────────────────────────

Document Version: 2.0

Last Updated: January 2025

Implementation: ai-service/app/api/endpoints/planning.py

Reference: Flask solar-battery-calculation endpoint